	EUROPEAN ORGANISATION FOR THE SAFETY OF AIR NAVIGATION


	[image: image7.wmf]E

U

R

O

C

O

N

T

R

O

L





	ANS

SOFTWARE

LIFECYLE




SAF.ET1.ST03.1000.REP-01-00

Edition
:
3.0

Edition Date
:
21/12/2005
Status
:
Released Issue

Class
:
General Public

	DOCUMENT IDENTIFICATION SHEET

	

	DOCUMENT DESCRIPTION

	Document Title

	ANS Software Lifecycle

	EWP DELIVERABLE REFERENCE NUMBER

	PROGRAMME REFERENCE INDEX
	EDITION :
	3.0

	SAF.ET1.ST03.1000.REP-01-00
	EDITION DATE :
	21/12/2005

	Abstract

	This document provides guidance material for defining an ANS software lifecycle. It also provides references to five existing standards (ED109, IEC12207, IEC61508, ED12B/DO178B and CMMi) and how these standards cover ANS needs.

	Keywords

	Software
	Safety Assurance
	Software Safety Assurance System
	

	Standards
	Quality Assurance
	SWAL
	

	Safety Assessment
	Assurance Level
	
	

	
	
	
	

	CONTACT PERSON :
	P.MANA
	TEL : 93295
	DIVISION :
	DAP/SAF

	

	DOCUMENT STATUS AND TYPE

	STATUS
	CATEGORY
	CLASSIFICATION

	Working Draft
	(
	Executive Task
	(
	General Public
	(

	Draft
	(
	Specialist Task
	(
	EATMP
	(

	Proposed Issue
	(
	Lower Layer Task
	(
	Restricted
	(

	Released Issue
	(
	
	
	
	

	

	ELECTRONIC BACKUP

	INTERNAL REFERENCE NAME :
	

	HOST SYSTEM
	MEDIA
	SOFTWARE(S)

	Microsoft Windows
	Type : Hard disk
	

	
	Media Identification : 
	


DOCUMENT APPROVAL

The following table identifies all management authorities who have successively approved the present issue of this document.

	AUTHORITY
	NAME AND SIGNATURE
	DATE

	Chairman of the EATMP Software Task Force
	P.MANA
	21/12/2005

	Chairman of the Safety Assessment Methodology Task Force
	P.MANA
	21/12/2005

	Chairman of the Safety Team
	E.MERCKX
	21/12/2005

	DAP Director
	G.PAULSON
	21/12/2005


DOCUMENT CHANGE RECORD

The following table records the complete history of the successive editions of the present document.

	EDITION
	DATE
	REASON FOR CHANGE
	SECTIONS PAGES
AFFECTED

	0.1
	11/06/1999
	First Issue
	All

	0.2
	10/12/1999
	Second Working Draft Issue, after review by the Task Force
	All

	0.3
	03/03/2000
	Third Working Draft Issue, after review by the Software Task Force
	All

	1.0
	29/10/2001
	Proposed Issue, after review by Software Task Force 
	All

	1.1
	23/11/201
	Proposed Issue after SWTF/4 meeting. The document title changed (old title: Software Standards Analysis)
	Introduction & Part I

	2.0
	25/04/2002
	Released Issue, after final review
	Part I

	3.0
	21/12/2005
	Second Released Issue, consistency with SAM V2.0, Recommendations for ANS SW V1.0, ESARR6 V1.0
	Part I


TABLE OF CONTENTS

DOCUMENT IDENTIFICATION SHEET
i

DOCUMENT APPROVAL
ii

DOCUMENT CHANGE RECORD
iii

TABLE OF CONTENTS
iv
Chapter 1- General Introduction
iiDOCUMENT APPROVAL


iiiDOCUMENT CHANGE RECORD


ivTABLE OF CONTENTS


11
PURPOSE


32 
SCOPE


33 
APPROACH


44
STRUCTURE OF THE CURRENT ISSUE


55
APPLICABILITY OF THE DOCUMENT


56
TARGET AUDIENCE


67
READERSHIP


78
HOW TO USE THIS DOCUMENT


CHAPTER 2- SOFTWARE STANDARDS OVERVIEW
91
INTRODUCTION


102
GENERAL PRESENTATION OF STANDARDS


122.1
ISO/IEC 12207


132.2
IEC 61508-3


152.3
ED-12B / DO-178B


172.4
ED 109/DO278


182.5
CMMI V1.1






[image: image1.wmf]1


GENERAL INTRODUCTION 

1
PURPOSE
An increasing proportion of ANS (Air Navigation System) functions is implemented by software and these functions are becoming more safety-critical. It is therefore necessary to define guidance on how assurance may be provided for software.

To complement the EATMP Air Navigation System Safety Assessment Methodology, initial material is needed for establishing such guidance and recommendations on the major activities required providing the appropriate safety and quality assurance level for software in Air Navigation Systems. 

A system throughout this document is composed of: people, procedure and equipment (Software, Hardware, Human Machine Interface (HMI)).

However today, no ANS software-related standard exists which neither fulfils ANS specificities (especially for ground part of ANS), nor is widely spread and extensively used by ANS community (at least not enough to become a de facto standard). 

Consequently, some standards have been chosen on which to base recommendations.

The objective of this document is not to promote any standard or to rank them. It just intends to identify the objectives/activities/tasks required by each standard and to describe their commonalities and differences.

The main objectives of this document are:

· to define an ANS software lifecycle
· to allow these different organisations to assess their own practices with respect to this recommended software lifecycle and to these standards.

The purposes of this document are:

· To define a recommended software lifecycle that matches ANS needs (Part I);

· To refer to existing standards developed for other domains of application (Part I);

· To assess the suitability of these standards for the definition, development, operation and maintenance of Air Navigation System software (Part II);

· To provide compatibility/traceability matrix between standards. For each process/activity of this recommended ANS software lifecycle, a reference will be provided to standards paragraphs that cover it either fully or partially (Part II);

· To provide for each of the five standards a coverage matrix, which identifies which processes/objectives of each standard are part of this recommended ANS software life (Part II);

· To provide the main omissions of these five standards as far as ANS needs are concerned.

2 
SCOPE

The software lifecycle described in this document applies to Air Navigation System Software. 

3 
APPROACH

As no safety and/or quality standard dedicated to ANS exists so far, the approach has been to perform a survey of existing software related standards what ever their domain of application.

Some safety-oriented standards exist such as ED12B/DO178B but which deals with airborne software in a certification environment or IEC 61508: a generic standard, which first requires to be tailored to a domain of application (this has not yet been done for ANS).

The selection of international standards is the following: 

· ISO/IEC 12207
Information Technology - Software Engineering - Software Life-Cycle Processes (November 1995).

· ED109 
Guidelines for Communication, Navigation, Surveillance, and Air Traffic Management (CNS/ ATM) Systems Software Integrity Assurance (March 2002)
· IEC 61508-3
Functional safety of electrical/electronic/programmable electronic safety-related systems
Part 3: Software Requirements (Draft Standard)

· ED12B/DO178B
Software Considerations in Airborne Systems and Equipment Certification (December 1992)

· CMMI  
Capability Maturity Model Integration (V1.1 March 2002)

Then an analysis of these standards and the identification of ANS specificities led to the definition of a ANS software lifecycle.

This ANS software lifecycle covers quality and safety related activities from the beginning of the system definition till decommissioning.

The intent of this document is not to define a new standard but to establish a reference against which to assess own practices. 
The approach elaborated relies on the analysis of best practices both from other domains using dedicated standards and also from ANS using the feedback of ANS stakeholders (regulatory bodies, ATS providers, industry, consultants, …).

4
STRUCTURE OF THE CURRENT ISSUE 

This current issue includes:

· Introduction:

· A general introduction identifying the purpose, scope, approach and content of this document (Chapter 1)

· A brief description of the five selected quality and safety standards for software development (Chapter 2). 

· Part I: ANS Software Lifecycle Definition

· An ANS software lifecycle is defined. This ANS software lifecycle is based on IEC/ISO 12207, but this does neither mean that this standard best fits ANS needs nor that it is the recommended one.

· A reference to those standards is provided. The purpose of this reference is to provide compatibility/traceability matrix between a recommended ANS software lifecycle and these standards. 

· The definition of the recommended ANS software lifecycle includes the following:

· Software Safety Assurance System (Chapter 1)

· Primary Lifecycle Processes (Chapter 2)

· Supporting Lifecycle Processes( Chapter 3)

· Organisational Lifecycle Processes (Chapter 4)

· Additional Software Lifecycle Objectives (Chapter 5)

· Part II: Software Standards Coverage

· This part identifies how each of the five standards is covered by the recommended ANS software lifecycle. Each standard paragraph or clause, which has been integrated in the ANS recommendations, is identified as such and a reference to the ANS recommendations paragraph is provided.

· This part also identifies the main omissions of these five standards as far as ANS particularities and the width of our scope are concerned.

5
APPLICABILITY OF THE DOCUMENT
This document recognises that the guidelines herein are not mandated by law, but represent a consensus of the ANS community on what are or should be the best practices.  

It also recognises that alternative methods to the methods described herein may be available to the stakeholders.  For these reasons, the use of words such as “shall’ and ‘must” is avoided, therefore all statements are using “should”.

6 TARGET AUDIENCE
This document is specifically targeted at:

Safety practitioners: 
Correct process in a methodologically correct way. 

They are responsible for:

the link between the programme/project and the safety assessment process, the methodological support to the different steps of the safety assessment process and the integration within the organisation Safety Management System (SMS). 

For example, the safety practitioners have to ensure that SWAL is allocated in accordance with Chapter 2, and that SWAL is validated.

Software Team:
Application in their domain knowledge. 

They use “ANS Software Lifecycle” to apply “Recommendations for ANS SW” for a specific software. 

For example, software team is responsible for the implementation of objectives of the allocated SWAL and for the verification & validation of their satisfaction. 

Project/Programme Manager or Safety Manager.

7 READERSHIP
The following table suggests a minimum reader’s attention to this document.

	
	Software Team
	Safety Practitioner
	Other roles (Programme/project Manager, Safety Manager, ..)

	Cover - 

Chapter 1 General Introduction


	(
	(
	(

	Cover - Chapter 2 –

SW Standards overview
	(
	(
	N/A

	Part I - Introduction
	(
	(
	(

	Part I - Chapter 1 –

Software Safety Assurance System
	(
	(
	(

	Part I - Chapter 2 –

Primary lifecycle
	(
	(
	(

	Part I - Chapter 3 –

Supporting Lifecycle
	(
	(
	(

	Part I - Chapter 4 –

Organisational Lifecycle
	(
	(
	(

	Part I - Chapter 5 –

Additional Lifecycle
	(
	(
	(

	Part II -Introduction

	(
	(
	N/A

	Part II – Chapter 1 – ED12B/DO178B

	(*
	(*
	(*

	Part II - Chapter 2 – IEC61508

	(*
	(*
	(*

	Part II - Chapter 3 – ISO/IEC 12207

	(*
	(*
	(*

	Part II - Chapter 4 – ED109/DO278

	(*
	(*
	(*

	Part II – Chapter 5 – 

CMMi

	(*
	(*
	(*


*: valid only for the standard being used by the organisation. Otherwise: N/A.
(: Detailed knowledge;

(: Aware;

N/A: Not Applicable.

8 HOW TO USE THIS DOCUMENT
This document can be used for the following purposes (see table example):

1. Identification of the ANS software lifecycle. The document user will have access to this reference lifecycle and its activities in the columns: 

· N°: activity Number;
· Activity Title: Reference name of the activity;

· Activity: Detailed description of the activity.

2. Assessment of its own practices. If the document user applies one of the five pre-assessed standards (ISO/IEC 12207, ED109, ED12B/DO178B, IEC61508 and CMMI), he/she will be able to compare directly his own practices with the reference lifecycle and its associated activities by reading the relevant column of the selected standard (the exact reference is provided):

· (  
(means fully covered: this standard proposes an equivalent activity); 

·  P 
(partially covered: this standard does not fully provide an equivalent activity); 

·  blank 
(missing: this standard does not provide an equivalent activity); 

3. Identification of activities (how) to satisfy an objective (what) listed in “Recommendations for ANS Software”. The document user will search the objective (Column Obj) to be satisfied for a specific SWAL and will find the proposed activities (Activity) to contribute to satisfy the objective (many activities could be necessary to satisfy one objective) and also how one of the 5 standards proposes to achieve this activity.
	N°
	Obj
	Activity Title
	Activity
	ISO/IEC 12207
	ED109
	ED-12B/

DO 178B
	IEC 61508
	
	CMMI

	2
	4.3.4
	Assurance Level Related Requirements
	Software requirements are commensurate with the allocated Assurance Level.
	
	(
(Ref: 3 

A2.1, A2.2)
	(
(Ref: 5.1.2, 11.9)
	(
(Ref: 7.2.2)
	
	P
(Ref: RD 3.3)

	3
	4.3.4, 4.3.15
	Software Requirements Definition Criteria
	The developer should specify & document  the software requirements considering the criteria listed below. 

a) Traceability to system requirements and system design;

b) External consistency with system requirements;

c) Internal consistency;

d) Testability;

e) Feasibility of software design;

f) Feasibility of operation and maintenance.
	(
(Ref: 5.3.4.2)
	(
(Ref: 3.3. Table  A2.1, A2.2 , A-3 line 6)
	(
(Ref: 5.5, 11.6, 11.9)
	P
(Ref: 7.2.2.1, 7.2.2.2, 

7.2.2.6)
	
	(
(Ref: RD 3.3
ReqM 1.4)

	4
	4.3.9, 4.3.10 4.3.11 4.3.12
	Software Requirements Standards
	Definition of methods, rules and tools to be used to develop software requirements.
	
	(
(Ref: 3.2 Tables  A2.1, A2.2) 
	(
(Ref: 11.6)
	(
(Ref: 7.2.2.4, 7.2.2.6)
	
	(
(Ref: RD GP 2.2, 2.3, 3.1)



[image: image2.wmf]2


SOFTWARE STANDARDS OVERVIEW 

1
INTRODUCTION

The purpose of this chapter is to provide a brief description of major software quality and/or safety standards.

This description intends to identify: 

· the organisation, which has defined the standard, 

· the scope of the standard, i.e. the list of processes and objectives or activities, which are to be performed during the lifecycle of the software development,

· the status of their use (industry, domain, …) and of their issue (recent,  to  be updated, …)

· safety-related considerations. 

2
GENERAL PRESENTATION OF STANDARDS

Five standards are considered in this document:

· ISO/IEC 12207
Information Technology - Software Engineering - Software Life-Cycle Processes (November 1995).

· ED109/DO278 
Guidelines for Communication, Navigation, Surveillance, and AIR TRAFFIC MANAGEMENT (CNS/ ATM) Systems Software Integrity Assurance (March 2002)

· IEC 61508-3
Functional safety of electrical/electronic/programmable electronic safety-related systems
Part 3: Software Requirements

· ED12B/DO178B
Software Considerations in Airborne Systems and Equipment Certification (December 1992)

· CMMI
Capability Maturity Model Integration (V1.1 March 2002)

Standards scope and their interrelationships are shown in Figure I.


[image: image3.wmf]PLANNING

DESIGN

CODING

INTEGRATION

SPECIFICATION

SUPPORTING PROCESS

SW PROJECT

DEVELOPMENT AND MAINTENANCE

ISO/IEC 12207

MIL-STD-498

SW DEPARTMENT

PURCHASE, SUPPLY, MAINTENANCE

ORGANIZATION

SW CRITICALITY

P

LANNING

SPE

CIFICATION

D

ESIGN

C

ODING

INTEGRATION

SÜPPORTING  PROCESS

SAFETY CRITICAL SW

DEVELOPMENT AND MAINTENANCE

ED12B/DO178B

IEC61508-3

ISO 9000-3

ISO-9000


Figure I. Scope and Interrelationships of Standards

The ISO/IEC 12207 Standard is currently considered as reflecting the best practices for all processes and activities of a Software lifecycle.

The IEC 61508-3 and the ED12B/DO178B cover the lifecycle of safety critical software. The IEC 61508-3 is part of an emerging generic standard (IEC 61508) addressing the functional safety of safety-related systems (in particular of the Equipment Under control (EUC), Cf: Annex A §2). This generic standard is expected to be tailored to a specific sector of application.

The EB12B/DO178B Standard defines recommended practices for the development of software in airborne systems and equipment. The Standard is not mandatory, but represents an international consensus in the avionics industry.

As explained in Chapter 1, only five standards have been considered further in the following document.

The MIL-STD-498 has been used in ANS industry. This standard is now superseded by the ISO/IEC 12207.

2.1
ISO/IEC 12207

This international Standard establishes a common framework for software lifecycle processes. The Standard specifies a comprehensive set of processes (described in terms of activities and tasks) covering all aspects of the software lifecycle. This international Standard groups the activities that may be performed during the lifecycle of software into five primary processes, eight supporting processes, and four organisational processes. These lifecycle processes are illustrated in Figure II.


[image: image4.wmf]PRIMARY

LIFE CYCLE PROCESSES

DEVELOPMENT

SECTION 5.3.

OPERATION

SECTION 5.4.

MAINTENANCE

SECTION 5.5.

SUPPLY

SECTION 5.2.

ACQUISITION

SECTION 5.1.

SUPPORTING

LIFE CYCLE PROCESSES

CONFIGURATION MANAGEMENT

SECTION 6.2.

DOCUMENTATION

SECTION 6.1.

QUALITY ASSURANCE

SECTION 6.3.

VERIFICATION

SECTION 6.4.

VALIDATION

SECTION 6.5.

JOINT REVIEW

SECTION 6.6.

AUDIT

SECTION 6.7.

PROBLEM RESOLUTION

SECTION 6.8.

IMROVEMENT

SECTION 7.3.

MANAGEMENT

SECTION 7.1.

INFRASTRUCTURE

SECTION 7.2.

TRAINING

SECTION 7.4.

ORGANIZATIONAL LIFE CYCLE PROCESSES


Figure II. Scope of ISO/IEC 12207 Standard

This international Standard is designed to be tailored for an individual organisation, project or application: an organisation, depending on its purpose, can select an appropriate subset to fulfil that purpose. In addition, the framework provides for controlling and improving these processes.

2.2
IEC 61508-3

The international Standard IEC 61508 sets out a generic approach for all safety lifecycle activities for systems comprised of electrical and/or electronic, and/or programmable electronic components (electrical/electronic/programmable electronic systems (E/E/PESs)).

This standard was originally designed based on the following principle:

A safety-related protection (monitoring) software controls an industrial process (EUC: Equipment Under Control), and can stop it. 

As a standard, it:

· is not sector specific 

· addresses a few design issues
· is primarily a process standard (mainly dedicated to safety management system, but not to “certify” or “qualify” or get approval for a product).
IEC61508 is a generic standard for all safety lifecycle activities for systems that are used to perform safety functions, which:

· provides a method for the development of the safety requirements specification necessary to achieve the required functional safety for safety-related systems

· adopts a broad range of principles, techniques and measures  to achieve functional safety
The standard consists of seven parts:

· Part 1: General requirements;

· Part 2: Requirements for electrical/electronic/programmable electronic systems (E/E/PES);

· Part 3: Software requirements;

· Part 4: Definitions and abbreviations;

· Part 5: Examples of methods for the determination of safety integrity levels;

· Part 6: Guidelines on the application of parts 2 and 3;

· Part 7: Overview of techniques and measures.

Relationships between the seven parts are shown in Figure III.


[image: image5.wmf]SYSTEM LIFE CYCLE PROCESS

DOCUMENTATION

MANAGEMENT

OF FUNCTIONAL

SAFETY

FUNCTIONAL

SAFETY

ASSESSMENT

PART 1

PART 2

HARDWARE LIFE

CYCLE PROCESS

PART 3

SOFTWARE LIFE

CYCLE PROCESS

PART 6

GUIDELINES FOR 

THE APPLICATION

OF PART 2 & 3

PART 7

OVERVIEW OF 

TECHNIQUES &

MEASURES

PART 5

EXAMPLES OF 

METHODS FOR 

THE 

DETERMINATION 

OF SAFETY 

INTEGRITY LEVELS

PART 4

DEFINITION

&

ABBREVIATION


Figure III. Structure of IEC 61508 Standard

Part III - Annex A of this document describes the general approach adopted in the IEC 61508.

Part 3 of IEC 61508 Standard describes the Software lifecycle activities. The software lifecycle aspects covered by the Standard are shown in Figure IV.

	SW SAFETY LIFE CYCLE REQUIREMENTS
	
	DOCUMENTATION

SECTION 5

	GENERAL

SECTION 7.1.
	
	

	SOFTWARE SAFETY REQUIREMENTS SPECIFICATION

SECTION 7.2.
	
	SW QUALITY MANAGEMENT SYSTEM

SECTION 6

	SOFTWARE SAFETY VALIDATION PLANNING

SECTION 7.3.
	
	

	SOFTWARE DESIGN AND DEVELOPMENT

SECTION 7.4.
	
	FUNCTIONAL SAFETY 

ASSESSMENT

SECTION 8

	PROGRAMMABLE ELECTRONICS INTEGRATION (HARDWARE AND SOFTWARE)

SECTION 7.5.
	
	

	SOFTWARE OPERATION AND MODIFICATION PROCEDURES

SECTION 7.6.
	
	

	SOFTWARE SAFETY VALIDATION

SECTION 7.7.
	
	

	SOFTWARE MODIFICATION

SECTION 7.8.
	
	

	SOFTWARE VERIFICATION

SECTION 7.9.
	
	

	
	
	

	GUIDE TO THE SELECTION OF TECHNIQUES AND MEASURES

ANNEX A
	
	

	
	
	

	DETAILED TABLES

ANNEX B
	
	


Figure IV: Scope of IEC 61508-3 Standard.

2.3
ED-12B / DO-178B

The purpose of ED12B/DO178B is to provide aviation airworthiness community with guidance for the production of software for airborne systems and equipment or with a level of confidence in safety that complies with airworthiness requirements.

The document describes the relationship between the system and software lifecycle, and between software development and the system safety assessment processes. However it does not address the system lifecycle, system safety assessment and validation processes. 

It is to be noted that no system safety assessment methodology (namely ARP4754 or ED79) was existing when ED12B/DO178B has been written.

Relationships between ED12B/DO178B and other documents developed by the airborne certification community are illustrated in Figure V. 


[image: image6.wmf]SYSTEM LIFE CYCLE PROCESS

(ARP 4754)

SAFETY ASSESSMENT PROCESS

GUIDELINES & METHODS

(ARP 4761)

SYSTEM DESIGN

FUNCTION FAILURES

& SAFETY INFORMATION

SOFTWARE LIFE

-

CYCLE

PROCESS

(ED

-

12B/DO

-

178B)

SOFTWARE LIFE

-

CYCLE

PROCESS

(ED

-

12B/DO

-

178B)

HARDWARE LIFE

-

CYCLE

PROCESS

(DO

-

254/ED

-

80)

HARDWARE LIFE

-

CYCLE

PROCESS

(DO

-

254/ED

-

80)

INTEGRATION

DESIGN

CONSTRAINTS

FUNCTIONAL

SYSTEM

INTENDED

FUNCTIONS


Figure V. Relationships between ED12B/DO178B and ARP documents

The scope of ED12B/DO178B is shown in Figure VI.




Figure VI: Scope of ED12B/DO 178B standard.

As the document addresses certification issues, it does not cover operational aspects of software. It does not address contractual relationships between the supplier and purchaser, nor the organisational aspects, competency criteria, and responsibility allocation of the supplier. 

The document refers to the concept of software lifecycle but does not prescribe the usage of a specific lifecycle model. 

It identifies six processes: 

· software planning, 

· software development, 

· software verification, 

· software configuration management,  

· software quality assurance and 

· software certification.

2.4
ED 109/DO278
This document provides guidelines for the assurance of software contained in non-airborne CNS/ATM systems. ED12B/DO178B, Software Considerations in Airborne Systems and Equipment Certification, defines a set of objectives that are recommended to establish assurance that airborne software has the integrity needed for use in a safety-related application. These objectives have been reviewed, and in some cases, modified for application to non-airborne CNS/ATM systems. This document is intended to be an interpretive guide for the application of ED-12B/DO-178B guidance to non-airborne CNS/ATM systems.
ED109/DO278 applies to software contained in CNS/ATM systems used in ground or space-based applications shown by a system safety assessment process to affect the safety of aircraft occupants or airframe in its operational environment.

The assurance of software resident within the airframe boundaries, including CNS/ATM-related equipment, is addressed by ED12B/DO178B.

A description of the prerequisite safety assessment process is not included in ED109/DO278.

Information on such assessments is available from other industry sources and in related regulatory guidance. Likewise, a complete description of the system life cycle processes, including system validation, as well as CNS/ATM systems approval, is not intended. ED109/DO278 is not intended to be a development standard nor a process document.
2.5
CMMI V1.1

The CMMI is a model developed by the Software Engineering Institute (SEI) of The Carnegie Mellon University. A large number of organizations from industry & US government have been involved in the development of this model.

As stated in the model, the purpose of this model is:

· to provide some guidance for an organisation to improve its processes,

· to serve as a reference to assess process capability/maturity level of the organization, and then to benchmark organizations.

The scope of this model covers the development, acquisition, and maintenance of product or services.

It may be used in various disciplines: System engineering, Software Engineering, Project Management and Supplier Sourcing. The extension to other disciplines (including safety engineering) is possible but requires a specific interpretation of the model to the discipline.

The CMMI is structured in “Process Areas” (PAs) and the maturity is defined in term of levels (from 0 or 1 up to 5). 

	Level
	Project Management PAs
	Engineering PAs
	Support PAs
	Process Management PAs

	5 Optimizing
	
	
	CAR: Causal Analysis & Resolution
	OID: Organizational Innovation & Deployment

	4 Quantitatively Managed
	QPM: Quantitative Project Management
	
	
	OPP : Organizational Process Performance

	3 Defined
	IPM: Integrated Project Management

RSKM: Risk Management 

IT: Integrated Teaming

ISM: Integrated Supplier Management
	RD: Requirements Development

TS: Technical Solution

PI: Product Integration

VER: Verification

VAL: Validation
	DAR: Decision Analysis & Resolution

OEI: Organizational Environment for Integration
	OPF: Organizational Process Focus

OPD: Organizational Process Definition

OT: Organizational Training

	2 Managed
	PP: Project Planning

PMC: Project Monitoring & Control

SAM: Supplier Agreement Management
	REQM: Requirements Management
	MA: Measurement & Analysis

PPQA: Process & Product Quality Assurance

CM: Configuration Management
	

	1 Initial
	
	
	
	


There are two representations of the model: staged or continuous.

The continuous representation is based on an independent levelling of each Process Area, whereas the staged representation is based on “global” levels, each level including both a set of pre-defined PAs and a common level for each of these processes. 

For example, using the continuous approach, an organization may be at level 2 for the Project Management PA, and at level 3 for Configuration Management PA, whereas using the staged model, if an organization is at level 3, all the level 3 goals of all the PAs pre-defined as belonging to the Staged Level 3 must be reached.

The levels (capability levels) in the continuous representation are the following: 

· Incomplete (0), 

· Performed (1), 

· Managed (2), 

· Defined (3), 

· Quantitatively Managed (4), and

· Optimizing (5).

The levels (maturity levels) in the staged representation are the following: 

· Initial (1), 

· Managed (2), 

· Defined (3), 

· Quantitatively Managed (4), and

· Optimizing (5).

Each Process Area includes a set of “goals”. Each goal is supposed to be reached by satisfying a set of requirements called “practices”. Goals & practices may be “specific” or “generic”. The “specific” goals and practices are dedicated to the Process Areas, whereas the “generic” ones are the same for all the PAs. For example, “Assign responsibility” or “Provide resources” are “generic”, i.e. applicable to any process. 

[image: image7.wmf]

This page is intentionally left blank.





































Verifying


Implementation





Directing


Implementation





Ability


to Perform





Commitment


to Perform





Specific Goals





Process Area n





Process Area 1





Common Features   - Generic Practices





Process Area 2





Generic Goals





Specific


Practices





Maturity Level








Page 17
Erreur ! Source du renvoi introuvable.
Edition : 3.0



EUROPEAN AIR TRAFFIC MANAGEMENT


_990363335.ppt






2








_1170849358.ppt






SYSTEM DESIGN

FUNCTION FAILURES

& SAFETY INFORMATION



SOFTWARE LIFE-CYCLE

PROCESS

(ED-12B/DO-178B)

HARDWARE LIFE-CYCLE

 PROCESS

(DO-254/ED-80)

INTEGRATION

DESIGN

CONSTRAINTS

FUNCTIONAL

SYSTEM

INTENDED

FUNCTIONS



SYSTEM LIFE CYCLE PROCESS

(ARP 4754)



SAFETY ASSESSMENT PROCESS

GUIDELINES & METHODS

(ARP 4761)








_1170853136.ppt






PART 1

PART 2



HARDWARE LIFE

CYCLE PROCESS



SYSTEM LIFE CYCLE PROCESS



DOCUMENTATION



MANAGEMENT

OF FUNCTIONAL

SAFETY



FUNCTIONAL

SAFETY

ASSESSMENT

PART 3



SOFTWARE LIFE

CYCLE PROCESS





PART 6



GUIDELINES FOR 

THE APPLICATION

OF PART 2 & 3



PART 7



OVERVIEW OF 

TECHNIQUES &

MEASURES





PART 5



EXAMPLES OF 

METHODS FOR 

THE 

DETERMINATION 

OF SAFETY 

INTEGRITY LEVELS



PART 4



DEFINITION

&

ABBREVIATION










_990363405.ppt






1








_990520970.doc


L







O







R







T







N







O







C







O







R







U







E












_979039703

_981978299.ppt








DEVELOPMENT

SECTION 5.3.



OPERATION

SECTION 5.4.



MAINTENANCE

SECTION 5.5.



SUPPLY

SECTION 5.2.



ACQUISITION

SECTION 5.1.



PRIMARY

LIFE CYCLE PROCESSES



CONFIGURATION MANAGEMENT

SECTION 6.2.



DOCUMENTATION

SECTION 6.1.



QUALITY ASSURANCE

SECTION 6.3.



VERIFICATION

SECTION 6.4.



VALIDATION

SECTION 6.5.



JOINT REVIEW

SECTION 6.6.



AUDIT

SECTION 6.7.



PROBLEM RESOLUTION

SECTION 6.8.



SUPPORTING

LIFE CYCLE PROCESSES







IMROVEMENT

SECTION 7.3.





MANAGEMENT

SECTION 7.1.







INFRASTRUCTURE

SECTION 7.2.







TRAINING

SECTION 7.4.



ORGANIZATIONAL LIFE CYCLE PROCESSES








_979022664

